| Issue | Date | Reason for Issue | Prepared |  | Checked |  | Approved |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Contents

1.	Highway Details	6

1.1	Type of highway	6

1.2	Permitted traffic speed	6

1.3	Existing restrictions	6

2.	Site Details	7

2.1	Obstacles crossed	7

3.	Proposed Structure	8

3.1	Description of structure and design working life	8

3.2	Structural type	8

3.3	Foundation type	8

3.4	Span arrangements	9

3.5	Articulation arrangements	9

3.6	Classes and levels	9

3.7	Road restraint systems requirements	10

3.8	Proposed arrangements for future maintenance and inspection	10

3.9	Environment and sustainability	11

3.10	Durability. Materials and Finishes	11

3.11	Risks and hazards considered for design, execution, maintenance and demolition.  Consultation with and/or agreement from CDM co-ordinator	12

3.12	Estimated cost of proposed structure together with other structural forms considered (including where appropriate proprietary manufactured structure), and the reasons for their rejection (including comparative whole life costs with dates of estimates)	13

3.13	Proposed arrangements for construction	13

4.	Design Criteria	16

4.1	Actions	16

4.2	Heavy or high load route requirements and arrangements being made to preserve the route, including any provision for future heavier loads or future widening	17

4.3	Minimum headroom provided	17

4.4	Authorities consulted and any special conditions required	17

4.5	Standards and documents listed in the Technical Approval Schedule	18

4.6	Proposed Departures relating to departures from standards given in 4.5	18

4.7	Proposed Departures relating to methods for dealing with aspects not covered by standards in 4.5	18

5.	Structural Analysis	19

5.1	Methods of analysis proposed for superstructure, substructure and foundations	19

5.2	Description and diagram of idealised structure to be used for analysis	19

5.3	Assumptions intended for calculation of structural element stiffness	19

5.4	Proposed range of soil parameters to be used in the design of earth retaining elements	19

6.	Geotechnical Conditions	20

6.1	Acceptance of recommendations of the Geotechnical Design Report to be used in the design and reasons for any proposed changes	20

6.2	Summary of design for highway structure in the Geotechnical Design Report	20

6.3	Differential settlement to be allowed for in the design of the structure	20

6.4	If the Geotechnical Design Report is not yet available, state when the results are expected and list the sources of information used to justify the preliminary choice of foundations	20

7.	Check	21

7.1	Proposed Category	21

7.2	If Category 3, name of proposed independent Checker	21

7.3	Erection proposals or temporary works for which Types S and P Proposals will be required, listing structural parts of the permanent structure affected with reasons	21

8.	Documents and Drawings	22

8.1	List of drawings (including numbers) and documents accompanying the submission	22

9.	Approvals	23

9.1	The Above is Submitted for Acceptance	23

9.2	The Above is Rejected/Accepted subject to the Amendments and Conditions Shown Below	23

Appendices

Appendix A Technical Approval Schedule

Appendix B General Arrangement Drawings

Appendix C Diagram of Idealisation of the Structure

Appendix D Design Risk Assessment

Appendix E Services Information

Appendix F Major Maintenance Costs

Appendix G Load Comparison Summary

Appendix H Bearing Manufacturer Statement

# Highway Details

## Type of highway

### Carrying

Public bridleway

### Crossing

A class Road

## Permitted traffic speed

### Carrying

Not applicable

### Crossing

50mph

## Existing restrictions

None

# Site Details

## Obstacles crossed

The A Class Road and vegetated cutting side slopes.

# Proposed Structure

## Description of structure and design working life

It is proposed to replace the existing concrete Wood Lane Footbridge (Client Council Structure No. 1234), which crosses the A Class Road with a new two span steel Warren truss footbridge.  The new steel truss superstructure will be supported by the existing reinforced concrete abutments and central pier.

In accordance with IAN 124/11 the design life for the bridge is Category 5 (120 years), with the exception of the movement joints, waterproofing systems, bearings and parapets, which are Category 2 (replaceable elements with up to 50 years design life).

The existing substructure will not be warranted for a 120 year design life. However as the substructure is less than 50 years old, it is expected to have a residual life of over 70 years.  In addition, any defects present in the substructure will be identified and repaired prior to the new bridge deck being installed.

To accommodate the widening of the A Class Road beneath the new Wood Lane Footbridge the existing earthworks cutting will be regraded over a length of approximately 65m.  The regrading will take advantage of the conjectured shallow depth to rock head to create an exposed rock cutting with a steepened slope angle which will be stabilised as required by a series of rock bolts.  The side slope angle of this rock cutting will be dependent on the nature and depth of the rock encountered on site.  Should the required steepening of the existing earthworks cutting not be achieved through the creation of an exposed rock face then the remaining earth retention shall be achieved through the installation of a reinforced concrete cantilever stem wall above the exposed rock face cutting.  A Geotechnical Design Report (GDR) will be submitted to the Client for approval for the regraded earthworks.  It is anticipated that any earth retaining wall situated above the exposed rock cutting will be less than 1.5m in height and as such will not be considered as a structure as defined by BD 02/12 of the Design Manual for Roads and Bridges.

## Structural type

A two span footbridge, comprising a welded half through steel Warren truss deck supported on existing and refurbished reinforced concrete bankseat type abutments and a central reinforced concrete pier.

## Foundation type

The north and south piers of the existing concrete Wood Lane Footbridge will be demolished.

Both existing bankseat abutments will be retained but modified to accommodate the new steel truss superstructure.  Modifications will include the breakout and reinstatement of the bearing shelfs and bearing plinths to provide the space to seat the new bearings required to articulate the superstructure.

The existing abutment ballast walls and cheek walls may also have to be demolished and reinstated to allow easier access to install the new bearings, to accommodate any change to the new bridge deck level and to aid in the installation of the new superstructure.

The existing central pier will also be retained and modified.  The top of both discrete concrete columns will be broken out and replaced with new reinforced concrete sections including a reinforced concrete cross beam which will provide a suitable bearing platform for the new steel truss superstructure.

Wherever possible continuity between old and new concrete sections shall be achieved through the mechanical breakout of areas of old concrete large enough to allow new reinforcement to be lapped on to suitably cleaned and prepared existing reinforcement.  Where this is not possible (due to restrictions in working space), or where it is preferable to use new precast members for elements such as the new central pier crosshead beam then connection shall be achieved through post drilled, resin anchored dowels with sufficient embedment into the existing concrete sections.

Prior to construction a dilapidation survey of all substructure elements to be retained will be undertaken.  All defects identified will be remedied through appropriate concrete repair techniques.

## Span arrangements

The structure will have a north span of 25.678m square (29.650m skew) and a south span of 24.898m square (28.750 skew), with a skew angle of 30 degrees to the A Class Road below.

## Articulation arrangements

The new footbridge will comprise two simply supported spans on elastomeric bearings fixed at the central pier.  The proposed bearings are Elastomeric Bearings, manufactured by Bearing Company.  These bearings are stated by the manufacturer to require no maintenance during their working life of 125 years (see manufacturers statement in Appendix H).  The bearings will be located inboard of the truss so that no elements of the bearing are directly exposed to the elements.

The design of the central pier will take due cognisance of the change to the bridge articulation from existing.  The fixed bearings will introduce additional horizontal loads at bearing level which will have to be translated down to the foundations to ensure globally and structurally the existing pier is capable of carrying the new bridge with its fixed articulation.  This is discussed in more detail in Section 5.

## Classes and levels

### Consequence class

The Wood Lane Footbridge will be designed to Consequence Class 2 in accordance with Table A.2 of IAN 124/11.

### Reliability class

Reliability class will be RC2 with K FI = 1.0 in accordance with Table A.2 of IAN 124/11.

### Inspection level

The inspection level will be IL2, in accordance Table A.2 of IAN 124/11.

## Road restraint systems requirements

- On Bridge Deck: 1800mm high equestrian containment parapets on either side of the structure in accordance with BS7818.  Parapets to have a solid infill for 1000mm minimum above the bridge deck and then vertical mesh infill above over the remaining 800mm height.

- On Road Below:  N2 safety barrier situated in the central reserve is to be retained.

## Proposed arrangements for future maintenance and inspection

The structure has been designed to reduce the requirements for future maintenance and inspection.

An outline method statement for a major maintenance painting scheme as described in Chapter 3 of BA28/92, together with an estimate of costs at 2015 rates is included in Appendix F.

### Traffic management

Partial closures of the carriageway will be required to inspect and maintain the bridge superstructure.

### Arrangements for future maintenance and inspection of structure.  Access arrangements to structure.

Mobile elevated working platforms will be required to access the underside of the bridge superstructure, the abutments and central pier.  It is envisaged that these platforms will be brought to site via the A class road.

A formal access route with suitable edge protection will be provided to afford access to the bridge bearing shelfs.

The proposed bearing replacement procedure is as follows:

Loosen the holding down bolts nuts;

Jack the structure up around 20mm at the bearing location;

Remove the existing bearing and place a replacement in its position;

Lower the structure down on the jack;

Retighten holding down bolts.

- Jacking locations will be marked on the construction drawings with jacking plates welded to the end transoms of the structure at these locations. The substructure will be designed to resist the anticipated jacking loads.

### Use of maintenance vehicles / special requirements

The bearing shelves and deck cross beams at support locations shall have provision for bearing replacement and deck jacking points.

The introduction of a new reinforced concrete cross beam between discrete columns in the central pier will provide space to for deck jacking points without the need for temporary works in the central reserve.

## Environment and sustainability

The proposed materials, protective treatments and finishes have been chosen to be fit for purpose, low toxicity to the environment and minimize the resources required.   The design re-uses as much of the existing substructure as is practicably possible reducing costs and minimising the use of new materials.

Noise pollution during the works is the primary environmental impact considered but can be mitigated through the Contractor’s considerate working practices including consultation with local residents.

A spray applied deck waterproofing system and anti-slip surfacing will also be specified.  The Contractor will ensure that these operations do not contaminate the local environment.

## Durability. Materials and Finishes

### Structural Concrete

In accordance with BS EN 206:2013, BS 8500-1:2006 and BS 8500-2:2006:

| Location | Concrete Class | Exposure Classes |
| --- | --- | --- |
| Modified abutment bearing shelfs, plinths and cheek walls | C40/50 | XD3, XC2 |
| Modified pier columns and cross head beam | C40/50 | XD3, XC2 |

### Structural Steelwork

Structural steelwork will be of Grade S355J2H to BS EN 10210-1:2006.

Structural steelwork will have an approved paint system with a minimum life of 25 years before major maintenance, colour to be agreed.

### Reinforcement

Steel reinforcement will be high yield type B500B or B500C ribbed bars to BS4449:2005.  Bars to be bent in accordance with BS 8666.

### Parapets

- Integral steel system.  Parapets to have a solid infill for 1000mm minimum above the bridge deck and then vertical mesh infill above over the remaining 800mm height.

### Movement joints and bearings

HA approved systems.  Upper faces of cover plates to expansion joints at deck level shall be provided with a suitable slip resistant coating.  The movement joints at the pier will be located such that they are not directly overhead of the bearings.

### Deck Waterproofing

Steel deck plate to be treated with a proprietary combined waterproofing and non-slip surfacing system.

### Finishes

Concrete Finishes will be as follows:

| Location/Description | Class (to SHW Series 1700) |
| --- | --- |
| Buried Unformed Surfaces | U1 |
| Buried/Hidden Formed Surfaces | F1 |
| Exposed Formed Faces | F3 |
| Exposed Unformed Faces | U3 |

All steelwork is to be painted in colour 12 B 29 in accordance with BS 4800.

### Impregnation and surface treatments

All buried surfaces shall receive a suitable primer layer and two coats of bituminous paint or approved equivalent in accordance with SHW Clause 2004.

All exposed concrete faces (existing and new) shall be impregnated with Pavix CCC100 or an equivalent approved system.

The proposed paint system topcoat for the whole structure shall be polysiloxane (International Paints item 979) which has graffiti-resistant properties.

## Risks and hazards considered for design, execution, maintenance and demolition.  Consultation with and/or agreement from CDM co-ordinator

A Design Risk Assessment can be found in Appendix D and will be developed further as the design progresses.  A residual risk register will be issued to the Contractor during the development of the construction information to ensure attention is drawn to any and all foreseen residual risks. The main risks envisaged are considered to be obvious to a competent Contractor experienced in carrying out such activities.

The demolition of the existing structure will need to be carefully managed to ensure uncontrolled failure does not occur.  Care will have to be taken to protect the substructure being retained from unwanted damage during demolition.

Consideration will also need to be given as to how the bridge will be maintained.  Temporary works required during operation such as bearing replacement will need to be protected from vehicular impact.  The introduction of a new pier cross head beam to jack the bridge from should minimise the need for any significant temporary works system being required during the lifetime of the structure.

Maintenance and inspection of the structure will require lane closures to the carriageway beneath.

The following risks have also been considered:

Demolition of existing bridge - destabilisation of existing slopes due to removal of existing piers;

Buried services and overhead services – to be protected and supported during construction or diverted away prior to work in that area;

Air pollution – control the dust in air during demolition and construction;

Noise pollution – particularly during demolition of existing concrete superstructure;

Fall from height during deck installation, inspection and future maintenance;

In Service hazards – Structure to be protected by VRS system.

In service hazards – safety of maintenance personnel during major maintenance such as painting works and bearing replacement.

## Estimated cost of proposed structure together with other structural forms considered (including where appropriate proprietary manufactured structure), and the reasons for their rejection (including comparative whole life costs with dates of estimates)

Design life is 120 years. Estimated cost is approx. £1.2m.  A two span structure provides the most efficient and cost effective solution for the length of span required. The re-use of the existing central pier reduces the materials required in the superstructure by 40% compared to an alternative single span solution.

## Proposed arrangements for construction

### Construction of structure

The anticipated construction sequence for the structure is as follows:

Site clearance and establish site access including traffic management and erection of safety barriers where required;

Demolition of existing bridge deck;

Demolition of the existing north and south piers;

Carry out concrete modifications required to the bankseat abutments which will include the breakout and reinstatement of the bearing shelfs and bearing plinths, existing ballast walls and cheek walls;

Carry out concrete modifications required to the central pier which will include the top of both discrete concrete columns being broken down and replaced with new reinforced concrete sections including a reinforced concrete crosshead beam which will provide a suitable bearing platform for the new steel truss superstructure;

Wherever possible continuity between old and new concrete sections shall be achieved through the mechanical breakout of areas of old concrete large enough to allow new reinforcement to be lapped on to suitably cleaned and prepared existing reinforcement.  Where this is not possible (due to restrictions in working space), or where it is preferable to use new precast members for elements such as the new central pier crosshead beam then connection shall be achieved through post drilled, resin anchored dowels with sufficient embedment into the existing concrete sections;

Install bearings;

Establish crane platform and truss set down area;

Take delivery of steel truss superstructure and splice together on site as required;

Lift new steel truss superstructure into place;

Install waterproofing/anti-slip surface system to the deck;

Install subsurface drainage;

Tie in works to the approach footways;

Finishes including signage, bollards and line markings;

Open bridge to public.

### Traffic management

Modifications of the existing concrete elements to be retained shall take place within single lane closures of the A class road.  The new bridge is to be lifted into position in a single closure of the A class road that is to be no longer than 24 hours on a date to suit the client.

The demolition of the existing bridge deck and north and south intermediate piers will be undertaken during a weekend closure of the A class road in both directions.  This will be agreed in advance with the relevant authorities.  The A class road surface will be suitably protected during the works to prevent damage to the A class road.

### Service diversions

At this stage, prior to confirmatory trial pits, it is envisaged that the following utilities are likely to be directly affected and will require diversion and or protection in place:

BT lines are enclosed within the existing deck cross section and will be diverted prior to the works.  Details of the proposed diversion method for the BT line is still in discussion at this stage.  Two options are being considered, a) direction drill option under the A class road, and b) temporary slewing of cables onto poles/supports and then final transfer into the new structure;

700mm diameter water main immediately adjacent to the north abutment is to remain in-situ and be protected as necessary during the works.

### Interface with existing structures

The existing Wood Lane Footbridge superstructure along with the north and south piers are to be demolished.  The existing bankseat abutments and central pier are to be retained and upgraded and refurbished as required.

# Design Criteria

## Actions

### Permanent actions

All permanent actions as defined in BS EN 1991-1-1:2002 and the National Annex.

Material densities and load factors for permanent actions will be taken from BS EN 1991-1-1:2002 and BS EN 1990:2002+A1:2005 respectively, and the associated UK National Annexes.

### Snow, Wind and Thermal actions

Snow actions will be considered in accordance with BS EN 1991-1-3:2003 and its UK National Annex.

Wind actions will be considered in accordance with BS EN 1991-1-4:2005 and its UK National Annex.

Thermal Actions will be applied in accordance with BS EN 1991-1-5:2003 and its UK National Annex.

### Actions relating to normal traffic under AW regulations and C&U regulations

Not applicable

### Actions relating to General Order traffic under STGO regulations

Not applicable

### Footway or footbridge variable actions

Footway live loading as defined in BS EN 1991-2:2003 and the National Annex.

Loading from a service vehicle shall be considered in accordance with BS EN 1991-2:2003 and clause NA.2.38 of the National Annex.  Horizontal forces from the service vehicle shall be considered in accordance with BS EN 1991-2:2003 and clause NA.2.39 of the National Annex.

### Actions relating to Special Order traffic, provision for exceptional abnormal  indivisible loads including location of vehicle track on deck cross-section

Not applicable

### Accidental actions

Actions for accidental design situations will be applied in accordance with EN 1991-2:2003.

The effects of vehicle impact on the bridge superstructure should be considered in accordance with EN 1991-2;2003, EN 1991-1-7:2003 and the relevant UK National Annexes along with the recommendations in PD 6688-1-7:2009.

### Action during construction

The stability of steel truss superstructure will be assessed in the temporary condition (i.e. during lifting operations and when jacked for bearing replacement) in accordance with BS EN 1991-1-6:2005 and its UK National Annex.

### Any special action not covered above

None.

## Heavy or high load route requirements and arrangements being made to preserve the route, including any provision for future heavier loads or future widening

Not applicable

## Minimum headroom provided

Minimum new construction headroom will be provided in accordance with TD 27/05, as implemented by IAN 124/11 i.e.: 5.70m plus sag curve compensation plus an allowance for settlement.  This headroom will be provided for the serviceability limit state.

Structure deflections and sag curve compensation shall be calculated in accordance with PD 6688-1-7:2009.

## Authorities consulted and any special conditions required

Statutory undertakers were consulted to ascertain the presence of plant. These consultations indicated there are the following buried services in the vicinity of the structure:

BT lines are enclosed within the existing deck cross section and will be diverted prior to the works;

700mm diameter water main immediately adjacent to the north abutment is to remain in-situ and be protected as necessary during the works.

A tracker document is included in Appendix E, showing the affected utilities.

| Authority Consulted | Special Requirements |
| --- | --- |
| Roads Authority | None. |
| Adjacent Local Authority | None. |
| Local Authority | Aesthetics: Photomontages of the Tenderers structural proposals shall be submitted with the proposed AIP for review and comment. The span of the structure is to be such that there is no substructure encroachment on the widened carriageway corridor. NOTE: Both of these requirements are to be discussed with the aim of removing these requirements from the design |
| Environment Agency | Design and construction to be in accordance with Flood Drainage Consent Process. |

## Standards and documents listed in the Technical Approval Schedule

See Appendix A

## Proposed Departures relating to departures from standards given in 4.5

No departures envisaged.

## Proposed Departures relating to methods for dealing with aspects not covered by standards in 4.5

None Applicable

# Structural Analysis

## Methods of analysis proposed for superstructure, substructure and foundations

The superstructure will be analysed using a 3D space frame model using STRAP or similar approved software analysis program.  Hand calculations assisted by Excel worksheets shall be adopted for post-processing analysis results.

The structure shall be checked for pedestrian induced vibration.

A load comparison assessment has been undertaken to confirm that the new superstructure does not induce more onerous loading conditions on the existing foundations being retained in the design.  A summary of this assessment can be found in Appendix G.

The new central pier arrangement (new cross beam and existing pier) will then be analysed using hand calculations assisted by Excel worksheets using reactions obtained from the 3D space frame model created for the design of the new superstructure.  This will include a detailed investigation of global stability of the pier under the new loading regime induced by the fixed bearings.  Global overturning and sliding checks will be undertaken in accordance with BS EN 1997 under the EQU, GEO/STR and Accidental limit states outlined in BS EN 1990.

Existing concrete sections will be analysed using Autodesk Structural Bridge Designer 2014 for the new applied loadings to confirm the adequacy of these sections.

Strengthened concrete sections will be treated as composite sections and analysed using Autodesk Structural Bridge Designer 2014 for the new loadings to confirm the adequacy of these sections.  Material properties for the existing concrete will be taken as per the record drawings for each stage of construction. In addition, longitudinal shear will be considered at the interface between the old and new sections to ensure that the column will act in a composite manner.

The existing abutments will be subjected to a quantitative assessment in accordance with BD 21/01.

## Description and diagram of idealised structure to be used for analysis

See Appendix C for the idealised model.

## Assumptions intended for calculation of structural element stiffness

Gross section properties will be used for all members for the global analysis.

## Proposed range of soil parameters to be used in the design of earth retaining elements

Not applicable

# Geotechnical Conditions

## Acceptance of recommendations of the Geotechnical Design Report to be used in the design and reasons for any proposed changes

A Geotechnical Report will not be prepared for this structure. See Section 0.

## Summary of design for highway structure in the Geotechnical Design Report

A Geotechnical Report will not be prepared for this structure. See Section 0.

## Differential settlement to be allowed for in the design of the structure

Differential settlement of 20mm is permitted

## If the Geotechnical Design Report is not yet available, state when the results are expected and list the sources of information used to justify the preliminary choice of foundations

A load comparison assessment has been undertaken to confirm that the new superstructure does not induce more onerous loading conditions on the existing foundations being retained in the design.  This can be found in Appendix G.

There is a slight increase in the maximum peak bearing pressure under the central pier foundations (approximately 17%), caused by the additional moments generated by the new horizontal loading applied to the top of the pier.  The peak bearing pressure is still significantly less than the allowable bearing pressure stated in the existing Wood Lane Bridge AIP of 400kPa (pier founded directly on to Pennant Sandstone) therefore this increase is considered acceptable.

# Check

## Proposed Category

The bridge is a Category II structure

## If Category 3, name of proposed independent Checker

Not applicable.

## Erection proposals or temporary works for which Types S and P Proposals will be required, listing structural parts of the permanent structure affected with reasons

None.

# Documents and Drawings

## List of drawings (including numbers) and documents accompanying the submission

Appendix A -Technical Approval Schedule

Appendix B - GA drawings

Drawing 100 – General Arrangement

Appendix C - Diagram of Idealised Structure

Appendix D – Design Risk Assessment

Appendix E - Services Information

Appendix F – Major Maintenance Costs

Appendix G – Load Comparison

Appendix H – Bearing Manufacturer Statement

# Approvals

## The Above is Submitted for Acceptance

### Designer

| Signed | ____________________________________________ |
| --- | --- |
| Name | ____________________________________________ |
|  | Design Team Leader |
| Engineering Qualifications | ____________________________________________ |
| Name of Organisation | ____________________________________________ |
| Date | ____________________________________________ |

### Contractor

| Signed | ____________________________________________ |
| --- | --- |
| Name | ____________________________________________ |
| Position Held | ____________________________________________ |
| Engineering Qualifications | ____________________________________________ |
| Name of Organisation | ____________________________________________ |
| Date | ____________________________________________ |

## The Above is Rejected/Accepted subject to the Amendments and Conditions Shown Below

| Signed | ____________________________________________ |
| --- | --- |
| Name | ____________________________________________ |
| Position held | ____________________________________________ |
| Engineering Qualifications | ____________________________________________ |
| TAA | ____________________________________________ |
| Date | ____________________________________________ |

Schedule of Documents Relating to Design of Highway Bridges and Structures

(All documents are taken to include revisions current as of 3 March 2015)

Eurocodes and associated UK National Annexes

|  | Eurocode part | Title | Amendment / Corrigenda |  |
| --- | --- | --- | --- | --- |
|  | Eurocode 0 | Basis of structural design |  |  |
|  | BS EN 1990 +A1:2005 | Eurocode 0: Basis of structural design | +A1:2005 Corrigenda December 2008 and April 2010 | See IAN 124/11 Annex A for additional guidance. |
|  | NA to BS EN 1990:2002 + A1:2005 | UK National Annex to Eurocode 0 Basis of structural design | National Amendment No.1 | See IAN 124/11 Annex A for additional guidance. |
|  | Eurocode 1 | Actions on structures |  |  |
|  | BS EN 1991-1-1:2002 | Eurocode 1: Actions on structures. General Actions. Densities, self-weight, imposed load for buildings | Corrigenda December 2004 and March 2009 |  |
|  | NA to BS EN 1991-1-1:2002 | UK National Annex to Eurocode 1: Actions on structures. General Actions. Densities, self-weight, imposed load for buildings | - |  |
|  | BS EN 1991-1-3:2003 | Eurocode 1: Actions on structures. General Actions. Snow loads | Corrigenda December 2004 and March 2009 |  |
|  | NA to BS EN 1991-1-3:2003 | UK National Annex to Eurocode 1: Actions on structures. General Actions. Snow loads | Corrigendum No.1 |  |
|  | BS EN 1991-1-4:2005 | Eurocode 1: Actions on structures. General Actions. Wind actions | +A1:2010 Corrigenda July 2009 and January 2010 |  |
|  | NA to BS EN 1991-1-4:2005 + A1:2010 | UK National Annex to Eurocode 1: Actions on structures. General Actions. Wind actions | National Amendment No.1 |  |
|  | BS EN 1991-1-5:2003 | Eurocode 1: Actions on structures. General Actions. Thermal actions | Corrigenda December 2004 and March 2009 |  |
|  | NA to BS EN 1991-1-5:2003 | UK National Annex to Eurocode 1: Actions on structures. General Actions. Thermal actions | - |  |
|  | BS EN 1991-1-6:2005 | Eurocode 1: Actions on structures. General Actions. Actions during execution | Corrigenda July 2008, November 2012 and February 2013 |  |
|  | NA to BS EN 1991-1-6:2005 | UK National Annex to Eurocode 1: Actions on structures. General Actions. Actions during execution | - |  |
|  | BS EN 1991-1-7:2006 | Eurocode 1: Actions on structures. General Actions. Accidental actions | Corrigendum February 2010 |  |
|  | NA to BS EN 1991-1-7:2006 | UK National Annex to Eurocode 1: Actions on structures. Part 1-7 : Accidental actions | - | See IAN 124/11 Annex A for additional guidance. |
|  | BS EN 1991-2:2003 | Eurocode 1: Actions on structures. Traffic loads on bridges | Corrigenda December 2004 and February 2010 | See IAN 124/11 Annex A for additional guidance. |
|  | NA to BS EN 1991-2:2003 | UK National Annex to Eurocode 1: Actions on structures. Traffic loads on bridges | Corrigendum No.1 | See IAN 124/11 Annex A for additional guidance. |
|  | Eurocode 2 | Design of concrete structures |  |  |
|  | BS EN 1992-1-1:2004 | Eurocode 2: Design of concrete structures– Part 1-1: General rules and rules for buildings | Corrigendum January 2008 and November 2010 |  |
|  | NA to BS EN 1992-1-1:2004 | UK National Annex to Eurocode 2: Design of concrete structures – Part 1-1: General rules and rules for buildings | National Amendment No.1 |  |
|  | BS EN 1992-2:2005 | Eurocode 2: Design of concrete structures – Part 2: Concrete bridges – Design and detailing rules | Corrigendum July 2008 |  |
|  | NA to BS EN 1992-2:2005 | UK National Annex to Eurocode 2: Design of concrete structure – Part 2: Concrete bridges – Design and detailing rules | - |  |
|  | BS EN 1992-3:2006 | Eurocode 2: Design of concrete structures – Part 3: Liquid retaining and containment structures | - |  |
|  | NA to BS EN 1992-3:2006 | UK National Annex to Eurocode 2: Design of concrete structure – Part 3: Liquid retaining and containment structures | - |  |
|  | Eurocode 3 | Design of steel structures |  |  |
|  | BS EN 1993-1-1:2005 | Eurocode 3: Design of steel structures – Part 1-1 General rules and rules for buildings | Corrigenda February 2006 and April 2009 |  |
|  | NA to BS EN 1993-1-1:2005 | UK National Annex to Eurocode 3: Design of steel structures – Part 1-1 General rules and rules for buildings | - |  |
|  | BS EN 1993-1-3:2006 | Eurocode 3: Design of steel structures – Part 1-3 General rules – Supplementary rules for cold-formed members and sheeting | Corrigendum November 2009 |  |
|  | NA to BS EN 1993-1-3:2006 | UK National Annex to Eurocode 3: Design of steel structures – Part 1-3 Supplementary rules for cold-formed members and sheeting | - |  |
|  | BS EN 1993-1-4:2006 | Eurocode 3: Design of steel structures – Part 1-4 General rules – Supplementary rules for stainless steels | - |  |
|  | NA to BS EN 1993-1-4:2006 | UK National Annex to Eurocode 3: Design of steel structures – Part 1-4 Supplementary rules for stainless steels | - |  |
|  | BS EN 1993-1-5:2006 | Eurocode 3: Design of steel structures – Part 1-5 Plated structural elements | Corrigendum April 2009 |  |
|  | NA to BS EN 1993-1-5:2006 | UK National Annex to Eurocode 3: Design of steel structures – Part 1-5 Plated structural elements | - |  |
|  | BS EN 1993-1-6:2007 | Eurocode 3: Design of steel structures – Part 1-6 Strength and stability of shell structures | Corrigendum April 2009 |  |
|  | BS EN 1993-1-7:2007 | Eurocode 3: Design of steel structures – Part 1-7 Plated structures subject to out of plane loading | Corrigendum April 2009 |  |
|  | BS EN 1993-1-8:2005 | Eurocode 3: Design of steel structures – Part 1-8 Design of joints | Corrigenda December 2005, September 2006, July 2009 and August 2010 |  |
|  | NA to BS EN 1993-1-8:2005 | UK National Annex to Eurocode 3: Design of steel structures – Part 1-8 Design of joints | - |  |
|  | BS EN 1993-1-9:2005 | Eurocode 3: Design of steel structures – Part 1-9 Fatigue | Corrigenda December 2005, September 2006 and April 2009 |  |
|  | NA to BS EN 1993-1-9:2005 | UK National Annex to Eurocode 3: Design of steel structures – Part 1-9 Fatigue | - |  |
|  | BS EN 1993-1-10:2005 | Eurocode 3: Design of steel structures – Part 1-10 Material toughness and through-thickness properties | Corrigenda December 2005, September 2006 and March 2009 |  |
|  | NA to BS EN 1993-1-10:2005 | UK National Annex to Eurocode 3: Design of steel structures – Part 1-10 Material toughness and through thickness properties | - |  |
|  | BS EN 1993-1-11:2006 | Eurocode 3: Design of steel structures – Part 1-11 Design of structures with tension components | Corrigendum April 2009 |  |
|  | NA to BS EN 1993-1-11:2006 | UK National Annex to Eurocode 3: Design of steel structures – Part 1-11 Design of structures with tension components | - |  |
|  | BS EN 1993-1-12:2007 | Eurocode 3: Design of steel structures – Part 1-12 Additional rules for the extension of EN 1993 up to steel grades S 700 | Corrigendum April 2009 |  |
|  | NA to BS EN 1993-1-12:2007 | UK National Annex to Eurocode 3: Design of steel structures – Part 1-12 Additional rules for the extension of EN 1993 up to steel grades S 700 | - |  |
|  | BS EN 1993-2:2006 | Eurocode 3: Design of steel structures – Part 2 Steel bridges | Corrigendum July 2009 |  |
|  | NA + A1:2012 to BS EN 1993-2:2006 | UK National Annex to Eurocode 3: Design of steel structures – Part 2 Steel bridges | + A1:2012 |  |
|  | BS EN 1993-5:2007 | Eurocode 3: Design of steel structures – Part 5 Piling | Corrigendum May 2009 |  |
|  | NA + A1:2012 to BS EN 1993-5:2007 | UK National Annex to Eurocode 3: Design of steel structures – Part 5 Piling | + A1:2012 |  |
|  | Eurocode 4 | Design of composite steel and concrete structures |  |  |
|  | BS EN 1994-1-1:2004 | Eurocode 4: Design of composite steel and concrete structures – Part 1-1 General rules and rules for buildings | Corrigendum April 2009 |  |
|  | NA to BS EN 1994-1-1:2004 | UK National Annex to Eurocode 4: Design of composite steel and concrete structures – Part 1-1 General rules and rules for buildings | - |  |
|  | BS EN 1994-2:2005 | Eurocode 4: Design of composite steel and concrete structures – Part 2 General rules and rules for bridges | Corrigendum July 2008 |  |
|  | NA to BS EN 1994-2:2005 | UK National Annex to Eurocode 4: Design of composite steel and concrete structures – Part 2 General rules and rules for bridges | - |  |
|  | Eurocode 5 | Design of timber structures |  |  |
|  | BS EN 1995-1-1:2004 + A1:2008 | Eurocode 5: Design of timber structures – Part 1-1 General – common rules and rules for buildings | + A1:2008 Corrigendum June 2006 |  |
|  | NA to BS EN 1995-1-1:2004 + A1:2008 | UK National Annex to Eurocode 5: Design of timber structures – Part 1-1 General – common rules and rules for buildings | + A1:2008 National Amendment No. 2 |  |
|  | BS EN 1995-2:2004 | Eurocode 5: Design of timber structures – Part 2 Bridges | - |  |
|  | NA to BS EN 1995-2:2004 | UK National Annex to Eurocode 5: Design of timber structures – Part 2 Bridges | - |  |
|  | Eurocode 6 | Design of masonry structures |  |  |
|  | BS EN 1996-1-1:2005 | Eurocode 6: Design of masonry structures – Part 1-1 General rules for reinforced and unreinforced masonry structures | Corrigenda February 2006 and July 2009 |  |
|  | NA to BS EN 1996-1-1:2005 | UK National Annex to Eurocode 6: Design of masonry structures – Part 1-1 General rules for reinforced and unreinforced masonry structures | - |  |
|  | BS EN 1996-2:2006 | Eurocode 6: Design of masonry structures – Part 2 Design considerations, selection of materials and execution of masonry | Corrigendum September 2009 |  |
|  | NA to BS EN 1996-2:2006 | UK National Annex to Eurocode 6: Design of masonry structures – Part 2 Design considerations, selection of materials and execution of masonry | Corrigendum No.1 |  |
|  | BS EN 1996-3:2006 | Eurocode 6: Design of masonry structures – Part 3 Simplified calculation methods for unreinforced masonry structures | Corrigendum October 2009 |  |
|  | NA to BS EN 1996-3:2006 | UK National Annex to Eurocode 6: Design of masonry structures – Part 3 Simplified calculation methods for unreinforced masonry structures | - |  |
|  | Eurocode 7 | Geotechnical design |  |  |
|  | BS EN 1997-1:2004 | Eurocode 7: Geotechnical design – Part 1 General rules | Corrigendum February 2009 |  |
|  | NA to BS EN 1997-1:2004 | UK National Annex to Eurocode 7: Geotechnical design – Part 1 General rules | Corrigendum No.1 |  |
|  | BS EN 1997-2:2007 | Eurocode 7: Geotechnical design – Part 2 Ground investigation and testing | Corrigendum June 2010 |  |
|  | NA to BS EN 1997-2:2007 | UK National Annex to Eurocode 7: Geotechnical design – Part 2 Ground investigation and testing | - |  |
|  | Eurocode 8 | Design of structures for earthquake resistance |  |  |
|  | BS EN 1998-1:2004 + A1:2013 | Eurocode 8: Design of structures for earthquake resistance – Part 1 General rules, seismic actions and rules for buildings | Corrigendum June 2009, January 2011 and March 2013 |  |
|  | NA to BS EN 1998-1:2004 | UK National Annex to Eurocode 8: Design of structures for earthquake resistance – Part 1 General rules, seismic actions and rules for buildings | - |  |
|  | BS EN 1998-2:2005+A2:2011 | Eurocode 8: Design of structures for earthquake resistance – Part 2 Bridges | Corrigenda February 2010 and February 2012 |  |
|  | NA to BS EN 1998-2:2005 | UK National Annex to Eurocode 8: Design of structures for earthquake resistance – Part 2 Bridges | - |  |
|  | BS EN 1998-5:2004 | Eurocode 8: Design of structures for earthquake resistance – Part 5 Foundations, retaining structures and geotechnical aspects | - |  |
|  | NA to BS EN 1998-5:2004 | UK National Annex to Eurocode 8: Design of structures for earthquake resistance – Part 5 Foundations, retaining structures and geotechnical aspects | - |  |
|  | Eurocode 9 | Design of aluminium structures |  |  |
|  | BS EN 1999-1-1:2007 + A1:2009 | Eurocode 9: Design of aluminium structures– Part 1-1 General structural rules | + A1:2009 |  |
|  | NA to BS EN 1999-1-1:2007 + A1:2009 | UK National Annex to Eurocode 9: Design of aluminium structures – Part 1-1 General structural rules | National Amendment No.1 Corrigendum No.1 |  |
|  | BS EN 1999-1-3:2007 + A1:2011 | Eurocode 9: Design of aluminium structures – Part 1-3 Structures susceptible to fatigue | + A1:2011 |  |
|  | NA to BS EN 1999-1-3:2007 + A1:2011 | UK National Annex to Eurocode 9: Design of aluminium structures – Part 1-3 Structures susceptible to fatigue | + A1:2011 |  |
|  | BS EN 1999-1-4:2007 +A1:2011 | Eurocode 9: Design of aluminium structures – Part 1-4 Cold formed structural sheeting | + A1:2011 Corrigendum November 2009 |  |
|  | NA to BS EN 1999-1-4:2007 | UK National Annex to Eurocode 9: Design of aluminium structures – Part 1-4 Cold formed structural sheeting | - |  |

Bsi Published Documents

|  | PD 6688-1-1:2011 | Recommendations for the design of structures to BS EN 1991-1-1 |
| --- | --- | --- |
|  | PD 6688-1-4:2009 | Background paper to the UK National Annex to BS EN 1991-1-4 |
|  | PD 6688-1-7:2009 | Recommendations for the design of structures to BS EN 1991-1-7 |
|  | PD 6688-2:2011 | Recommendations for the design of structures to BS EN 1991-2 |
|  | PD 6687-1:2010 | Background paper to the UK National Annexes to BS EN 1992-1 and BS EN 1992-3 |
|  | PD 6687-2:2008 | Recommendations for the design of structures to BS EN 1992-2:2005 |
|  | PD 6695-1-9:2008 | Recommendations for the design of structures to BS EN 1993-1-9 |
|  | PD 6695-1-10:2009 | Recommendations for the design of structures to BS EN 1993-1-10 |
|  | PD 6695-2:2008 + A1:2012 Incorporating Corrigendum No.1 | Recommendation for the design of bridges to BS EN 1993 |
|  | PD 6696-2:2007 + A1:2012 | Background paper to BS EN 1994-2 and the UK National Annex to BS EN 1994-2 |
|  | PD 6694-1:2011 | Recommendations for the design of structures subject to traffic loading to BS EN 1997-1 |
|  | PD 6698:2009 | Recommendations for the design of structures for earthquake resistance to BS EN 1998 |
|  | PD 6703:2009 | Structural bearings – Guidance on the use of structural bearings |
|  | PD 6705-2:2010 + A1:2013 | Recommendations for the execution of steel bridges to BS EN 1090-2 |
|  | PD 6705-3:2009 | Recommendations on the execution of aluminium structures to BS EN 1090-3 |
|  | PD 6702-1:2009 | Structural use of aluminium. Recommendations for the design of aluminium structures to BS EN 1999 |

Execution Standards referenced in British Standards or Eurocodes

|  | BS EN 1090-1:2009+A1:2011 | Execution of steel structures and aluminium structures - Part 1: Requirements for conformity assessment of structural components |
| --- | --- | --- |
|  | BS EN 1090-2:2008+A1:2011 | Execution of steel structures and aluminium structures – Part 2: Technical requirements for the execution of steel structures |
|  | BS EN 1090-3:2008 | Execution of steel structures and aluminium structures – Part 3: Technical requirements for aluminium structures |
|  | BS EN 13670:2009 | Execution of concrete structures |

Product Standards referenced in British Standards or Eurocodes

|  | BS EN 206-1:2000 | Concrete – Part 1: Specification, performance, production and conformity |
| --- | --- | --- |
|  | BS EN 206-9:2000 | Concrete – Part 9: Additional Rules for Self-compacting Concrete (SCC) |
|  | BS EN 1317-1:2010 | Road Restraint Systems – Part 1 – Terminology and general criteria for test methods |
|  | BS EN 1317-2:2010 | Road Restraint Systems – Part 2 – Performance classes, impact test acceptance criteria and test methods for safety barriers. |
|  | BS EN 1317-3:2010 | Road Restraint Systems – Part 3 – Performance classes, impact test acceptance criteria and test methods for crash cushions. |
|  | DD ENV 1317-4:2002 | Road Restraint Systems – Part 4 – Performance classes, impact test acceptance criteria and test methods for terminals and transitions of safety barriers. |
|  | BS EN 1317-5:2007+A2:2012 | Road Restraint Systems – Part 5 - Product requirements and evaluation of conformity for vehicle restraint systems |
|  | PD CEN/TR BS EN 1317-6:2012 | Road Restraint Systems – Part 6 – Pedestrian restraint system: Pedestrian parapets |
|  | BS EN 1337-1:2000 | Structural bearings – Part 1: General Design Rules |
|  | BS EN 1337-2:2004 | Structural bearings – Part 2: Sliding elements |
|  | BS EN 1337-3:2005 | Structural bearings – Part 3: Elastomeric bearings |
|  | BS EN 1337-4:2004 | Structural bearings – Part 4: Roller bearings |
|  | BS EN 1337-5:2005 | Structural bearings – Part 5: Pot bearings |
|  | BS EN 1337-6:2004 | Structural bearings – Part 6: Rocker bearings |
|  | BS EN 1337-7:2004 | Structural bearings – Part 7: Spherical and cylindrical PTFE bearings |
|  | BS EN 1337-8:2007 | Structural bearings – Part 8: Guide bearings and restraint bearings |
|  | BS EN 1337-9:1998 | Structural bearings – Part 9: Protection |
|  | BS EN 1337-10:2003 | Structural bearings – Part 10: Inspection and maintenance |
|  | BS EN 1337-11:1998 | Structural bearings – Part 11: Transport, Storage and Installation. |
|  | BS EN 1794-1:2011 | Road traffic noise reducing devices – Non-acoustic performance Part 1: Mechanical performance and requirements |
|  | BS EN 1794-2:2011 | Road traffic noise reducing devices – Non-acoustic performance Part 2: General safety and environmental requirements |
|  | BS EN 10025-1:2004 | Hot rolled products of structural steels Part 1: General technical delivery conditions. |
|  | BS EN 10025-2:2004 | Hot rolled products of structural steels Part 2: Technical delivery conditions for non-alloy structural steels. |
|  | BS EN 10025-3:2004 | Hot rolled products of structural steels Part 3: Technical delivery conditions for normalized/normalized rolled weldable fine grain structural steels. |
|  | BS EN 10025-4:2004 | Hot rolled products of structural steels Part 4: Technical delivery conditions for thermomechanical rolled weldable fine grain structural steels. |
|  | BS EN 10025-5:2004 | Hot rolled products of structural steels – Part 5: Technical delivery conditions for structural steels with improved atmospheric corrosion resistance |
|  | BS EN 10025-6:2004+A1:2009 | Hot rolled products of structural steels – Part 6: Technical delivery conditions for flat products of high yield strength structural steels in the quenched and tempered condition. |
|  | BS EN 10080:2005 | Steel for the reinforcement of concrete – Weldable reinforcing steel - General |
|  | BS EN 10210-1:2006 | Hot finished structural hollow sections of non-alloy and fine grain steels – Part 1: Technical delivery conditions |
|  | BS EN 10210-2:2006 | Hot finished structural hollow sections of non-alloy and fine grain steels – Part 2: Tolerances, dimensions and sectional properties |
|  | BS EN 10248-1:1996 | Hot rolled sheet piling of non alloy steels. Technical delivery conditions |
|  | BS EN 10248-2:1996 | Hot rolled sheet piling of non alloy steels. Tolerances on shape and dimensions |
|  | BS EN 12063:1999 | Execution of special geotechnical work. Sheet pile walls. |
|  | BS EN 15050:2007 + A1:2012 | Precast concrete products – Bridge elements |
|  | BS EN 14399-3:2005 | High strength structural bolting assemblies for preloading. System HR – Hexagon bolt and nut assemblies |
|  | BS EN 14399-10:2005 | High strength structural bolting assemblies for preloading. System HRC – Bolt and nut assemblies with calibrated preload |

British Standards

|  | BS 4449:2005+A2:2009 | Steel for the reinforcement of concrete |
| --- | --- | --- |
|  | BS 5896:2012 | Specification for high tensile steel wire and strand for the prestressing of concrete |
|  | BS 8006-1:2010 | Code of practice for strengthened/reinforced soils and other fills |
|  | BS 8500-1:2006 +A1:2012 Incorporating Corrigendum No.1 | Concrete – Complementary British Standard to BS EN 206-1 – Part 1: Method of specifying and guidance for the specifier. |
|  | BS 8500-2:2006 +A1:2012 Incorporating Corrigendum No.1 | Concrete – Complementary British Standard to BS EN 206-1 – Part 2: Specification for constituent materials and concrete. |

The Manual Contract Document for Highway Works (MCHW)

|  | MCHW Volume 1: November 2009 | Specification for Highway Works |
| --- | --- | --- |
|  | MCHW Volume 2: November 2009 | Notes for guidance on the Specification for Highway Works |
|  | MCHW Volume 3: November 2008 | Highway Construction Details |

The Design Manual for Roads and Bridges (DMRB)

|  | BD 2/12 | Technical Approval of Highway Structures |
| --- | --- | --- |
|  | BD 7/01 | Weathering steel for highway structures |
|  | BD 10/97 | Design of highway structures in areas of mining subsidence |
|  | BD 12/01 | Design of corrugated steel buried structures with spans greater than 0.9 metres and up to 8.0 metres |
|  | BD 21/01 | The Assessment of Highway Bridges and Structures |
|  | BD 29/04 | Design criteria for footbridges |
|  | BD 33/94 | Expansion joints for use in highway bridge decks |
|  | BD 35/06 | Quality assurance scheme for paints and similar protective coatings |
|  | BD 36/92 | Evaluation of maintenance costs in comparing alternative designs for highway structures |
|  | BD 43/03 | The impregnation of reinforced and prestressed concrete highway structures using hydrophobic pore-lining impregnants |
|  | BD 44/95 | The Assessment of Concrete Highway Bridges and Structures |
|  | BD 45/93 | Identification markings of highway structures |
|  | BD 47/99 | Waterproofing and surfacing of concrete bridge decks |
|  | BD 51/98 | Portal and cantilever signs/signal gantries |
|  | BD 57/01 | Design for durability |
|  | BD 62/07 | As built, operational and maintenance records for highway structures |
|  | BD 65/97 | Design criteria for collision protection beams |
|  | BD 67/96 | Enclosure of bridges |
|  | BD 68/97 | Crib retaining walls |
|  | BD 78/99 | Design of road tunnels |
|  | BD 82/00 | Design of buried rigid pipes |
|  | BD 90/05 | Design of FRP bridges and highway structures |
|  | BD 94/07 | Design of minor structures |
|  | BA 16/97 | The Assessment of Highway Bridges and Structures |
|  | BA 26/94 | Expansion joints for use in highway bridge decks |
|  | BA 28/92 | Evaluation of maintenance costs in comparing alternative designs for highway structures |
|  | BA 36/90 | The use of permanent formwork |
|  | BA 41/98 | The design and appearance of bridges |
|  | BA 42/96 | The design of integral bridges |
|  | BA 44/96 | The Assessment of Concrete Highway Bridges and Structures |
|  | BA 47/99 | Waterproofing and surfacing of concrete bridge decks |
|  | BA 55/06 | The Assessment of Bridge Substructures and Foundations, Retaining Walls and Buried Structures |
|  | BA 57/01 | Design for durability |
|  | BA 59/94 | Design of highway bridges for hydraulic action. |
|  | BA 67/96 | Enclosure of bridges |
|  | BA 68/97 | Crib retaining walls |
|  | BA 82/00 | Formation of continuity joints in bridge decks |
|  | BA 84/02 | Use of stainless steel reinforcement in highway structures |
|  | BA 85/04 | Coatings for concrete highway structures & ancillary structures |
|  | BA 92/07 | Use of recycled concrete aggregates in structural concrete |
|  | TD 19/06 | Requirement for road restraint systems |
|  | TD 27/05 | Cross-sections and headrooms |
|  | HD 22/08 | Managing geotechnical risk |
|  | HA 66/95 | Environmental barriers |

Interim Advice Notes

|  | IAN 41/02 | European cement standards |
| --- | --- | --- |
|  | IAN 47/02 | Post tensioned grouted duct concrete bridges |
|  | IAN 48/03 | Measures to minimise the risk of sulphate attack (including thaumasite) – New construction and structures under construction. |
|  | IAN 69/05 | Designing for maintenance |
|  | IAN 70/06 | Implementation of new reinforcement standards (BS 4449:2005, BS4482:2005, BS4483:2005 and BS8666:2005) |
|  | IAN 83/06 | Principal and General Inspection of Sign/Signal Gantries, and Gantries with low handrails or open mesh flooring |
|  | IAN 85/07 | Design of passively safe portal signal gantries. |
|  | IAN 86/07 | Amendments to the design requirements for portal and cantilever sign/signal gantries |
|  | IAN 95/07 | Revised guidance regarding the use of BS8500(2006) for the design and construction of structures using concrete. |
|  | IAN96/07r1 | Guidance on implementing results of research on bridge deck waterproofing. |
|  | IAN 97/07 | Assessment and upgrading of existing parapets |
|  | IAN 104/07 | The anchorage of reinforcement and fixings in hardened concrete. |
|  | IAN 105/08 | Implementation of construction (design and management) 2007 and the withdrawal of SD 10 and SD 11 |
|  | IAN 117/08r2 | Certification of combined kerb and drainage products |
|  | IAN 124/11 | Eurocodes: Implementation of Eurocodes for the design of new and existing highway structures |
|  | IAN 127/10r1 | The use of foamed concrete |
|  | IAN 131/11 | Deflection of Permanent Formwork |
|  | IAN 136/10 | Structural safety reporting |
|  | IAN 149/11 | Existing Motorway Minimum Requirements |
|  | IAN 161/12 | Managed Motorway Requirements – All Lane Running |
|  | IAN 177/13 | Introduction of the Construction Products Regulation (EU) 305/2011 |

Miscellaneous

|  | BRE Special Digest 1: 2005: Third Edition | Concrete in aggressive ground. |
| --- | --- | --- |
|  | CHE Memorandum 227/08 | The Impregnation of Reinforced and Prestressed Concrete Highway Structures using Hydrophobic Pore Lining Impregnants |
|  | CIRIA C543 | Bridge Detailing Guide |
|  | CIRIA C580 | Embedded Retaining Walls – Guidance for Economic Design |
|  | CIRIA C660 | Early-age Thermal Crack Control in Concrete |
|  | CIRIA C686 | Safe Access for Maintenance and Repair |